2022.7.3	Hobbs 2042.2	Front tire assembly removed for replacement.

Rim 765253 and hub 85225 were cleaned and inspected. No wear found.

Removal of the tire from the rim was done by removing the bolts that hold the two pieces together and break the bead the tire has to the rim.

Assembly: A paper gasket was placed between the two hub pieces to avoid pinching the tube.

Cheng Shin Tire 13 x 5.00 – 6 Serial: 1001386404 was cleaned and inspected for wear. I estimate 50% wear to the tire